GPIO Pulse Output

Version 1.0

Date:09/08/2021

Compac Industries Ltd.
52 Walls Road. Penrose. Auckland 1061. New Zealand.
PO Box 12 417. Penrose. Auckland 1642. New Zealand.
Tel: +64 9 579 2094   Fax: +64 9 579 0635   info@compac.co.nz

www.compac.co.nz

### Overview

The Pulse output is designed to interface the Compac dispenser to a 3 party Controller/POS without the need to talk a communication protocol.

The Pulse output has the following parameters that can be changed

Frequency

Duty Cycle

Output pulses for volume or amount

The Value of a pulse

### GPIO K Factor settings

The GPIO settings in the K factor board is where you set the GPIO specific settings. The below figure shows details of all the options available for each setting.

GPi0  
  0000

Inputs Setting
0-0 Off
1-1 Channel Encoder
2-2 Channel Encoder
3-3 Channel Encoder
4-Switch Input

Output Settings
0-0 off
1-Volume (Litres/Kg’s)
2-Amount (Dollars)

Inputs Setting
0-1KHz
1-100Hz
2-200Hz
3-300Hz
4-400Hz
5-500Hz
6-600Hz
7-700Hz
8-800Hz
9-900Hz
a-1KHz
b-1.1KHz
C-1.2KHz
d-1.3KHz
C-1.4KHz
d-1.4KHz

Duty Cycle Setting
0-50%
1-10%
2-20%
3-30%
4-40%
5-50%
6-60%
7-70%
8-80%
9-90%

### Output

The output setting enables the GPIO board to output pulses. It also sets whether the output pulses are representing volume(litre/Kg’s) or amount(dollars). Majority of applications will set the pulses to represent volume

Note you cannot have meter input enabled at the same time.

### Frequency

The Frequency output setting sets the maximum speed of the output pulses. The Default setting of 0 sets the frequency to 1KHz. This setting combined with the Value per pulse setting sets the maximum flow rate of the dispenser.

Example

- Frequency set to 1KHz (1000 pulses/sec)

- Pulses per value are set to 00001 (10ml/pulse)

That means the maximum flow rate the dispenser can do before the output pulses lag behind is

maximum flow=maxium frequency × pulses per value 
maximum flow=1000Hz × 10ml 
maximum flow=10000 ml per second
maximum flow=600 l per minute

The default setting of 1KHz should be sufficient for most applications. In applications where the maximum flow rate is lower and the 3rd party controller is only able to read pulses at a lower frequency then a lower frequency output can be selected.

Note that if the flow rate exceeds the maximum pulse output the next transaction will not be allowed until the pulses have completed being outputted.

### Duty Cycle

The duty cycle setting gives the ability to set the percentage of the pulse high and low. The default setting is 50%. The pulse length is determined by the following formula.

Example

- Duty cycle set to 50%

- Frequency set to 1KHz (1000 pulses/sec)

Duty Cycle=Pulse Width sec×Frequency Hz×100
50= Pulse Width sec ×1000×100
50100=Pulse Width sec ×1000
0.51000=Pulse Width sec 
500 microseconds=Pulse Width sec

The Default setting of 50% should be sufficient for most applications.

### Pulse value

The pulse value setting sets what a pulse is worth. When the output is set to volume the lowest volume a pulse can be set to is 00001 which is 0.1 ml. If the output is set to amount the lowest amount a pulse can be set to is 00001 which is 0.0001 dollars.

GPi0   pu 
  00000

The most common setting for most applications would be 00100 or a factor of 10. Any other setting would cause an error with rounding.

Pulse Output Interface

The pulse output is provided as an isolated open-collector transistor output via the following terminals:

- OUT – Open-collector pulse output

- ISO_GND – Isolated signal ground

- +V – Optional 5 V auxiliary output for pulse interface applications

Connection

The pulse output transistor switches the OUT terminal to ISO_GND when a pulse is generated.

Common Input

Receiving Device (PLC, Counter, RTU, etc.)

Using a Pull-Up Resistor

The pulse output is a current-sinking (open-collector) output. The receiving device must provide a pull-up voltage, or an external pull-up resistor must be fitted.

Example:

Receiving Device Input 5 Volts 4.7 kΩ Common

Using the Internal 5 V Supply

Receiving Device Common Input 4.7 kΩ + 12 VDC

Using an External 12 V Supply

Receiving Device Common Input 10 kΩ + 24 VDC

Using an External 24 V Supply

Recommended Pull-Up Resistor Values

| Pull-Up Voltage | Recommended Resistor |
| --- | --- |
| 5 V | 4.7 kΩ |
| 12 V | 4.7 kΩ |
| 24 V | 10 kΩ |

Pull-Up Resistor Rating

Pull-up resistor: Use a resistor rated at ¼ W (0.25 W) minimum. Higher power ratings, such as ½ W or 1 W, may also be used and will not affect operation.

Operation

- Transistor OFF: OUT is open circuit and the pull-up resistor holds the signal HIGH.

- Transistor ON: OUT is connected to ISO_GND and the signal is pulled LOW.

- Each LOW transition corresponds to a pulse from the dispenser.

Notes

- The pulse output is an open-collector output and does not source current.

- ISO_GND must be used as the signal return for the pulse output circuit.

- The +V terminal provides an optional 5 V supply for low-voltage pulse interfaces.

- If the receiving device already provides an input pull-up or wetting voltage, no external pull-up resistor is required.

- Where an external pull-up resistor is required, use the values listed above with a resistor rated at ¼ W or greater.

- Ensure the voltage applied to the pulse output does not exceed the maximum output rating specified for the device.